VLS-Protocol

Revision History

| Version/Author/Date | Revision Content |
| --- | --- |
| 1.0/Lei Chaoqun/2026-07-24 | First issue |

Table of Contents

1. Protocol Overview	3

1.1 Design Goals	3

1.2 Basic Communication Constraints	3

2. Basic General Specification	3

2.1 Unique bus Topic	3

2.2 Global public header	3

2.3 Generic Unified Receipt Template	4

3. Device Class	5

3.1 Full device configuration	5

3.2 General device control	6

3.3 Remote Snapshot	7

3.4 Device Calibration Time	8

3.5 Media Upload Interaction	9

3.6 Device heartbeat & Hardware telemetry	11

3.8 QR Code Recognition Report	13

3.10 Local Video Query	13

3.11 Video upload task release	15

3.13 Hardware information query	15

3.14 Device Log Paginated Report	17

4. AI Business type	18

4.1 AI Model Issue Deployment	18

4.2 Query current loading model	19

4.3 Model Manual Rollback	20

4.4 Face Library People Management	20

4.5 Face passage / Stranger Snap Report	21

4.6 Structured human carriage / Plate Number / Non-motor vehicle identification	23

5. IoT Center Type	24

5.1 RSGet	24

5.2 RSSave	25

5.3 Elevator control	26

5.4 LCD Advertising Management	27

5.5 MP3 Audio Playback Configuration	28

6. Global Reliability & General Safety Specifications	29

1. Protocol Overview

1.1 Design Goals

  - Interoperability: Compatible with standard IPC. PTZ cameras  PTZ. Face Recognition All-in-One. Structured Smart Camera. and all categories of edge devices, Unified interaction paradigm;

  - scalability: Adopt a hierarchical message structure, Business payloads are independently isolated, Add AI events. Device Configuration. Media uploads do not require modification of the outer protocol header;

  - Convenience: Reuse the existing HTTP backend API fields, Use one integration flow on the device HTTP/MQTT for both channels, Reduced retrofit costs;

  - Reliability: Perfect idempotent. Try again. Acknowledgment. Offline Cache. Heartbeat Mechanism, Adapting to Weak Network Industrial Scenarios;

  - security compliance: Device Independence ACL. TLS encryption. Message Authentication. Sensitive Field Desensitization.

1.2 Basic Communication Constraints

| Item | Specification Definition |
| --- | --- |
| MQTT Version | 3.1.1 (Compatible 5.0) |
| Production Port | TLS 8883; Test plaintext 1883 |
| ClientID | vlstream-{deviceId} Global Unique |
| KeepAlive | 60s |
| CleanSession | false (Persistent Sessions, Offline messages don't get lost) |
| QoS Unified | All Commands / events / Receipt Usage QoS 1 |
| Retain | Business Messages false; Device Online Status true |
| <g id="Bold">Code</g> | Full message UTF-8 JSON |
| Time standards | UTC ISO-8601 yyyy-MM-ddTHH:mm:ssZ |
| Idempotent rule | messageId UUID Global Unique, Retry unchanged |
| Binary Strategies | MQTT Pass only URL / Metadata, Images / Models / Video walk HTTP/MinIO Upload it, Do Base64 Large Package |

2. Basic General Specification

2.1 Unique bus Topic

vlstream/v2.2/dev/{deviceId}/bus

Full Business Multiplexing

Issued on all platforms. Device reports. Uniform release of business receipts /  SUBSCRIBE;

ACL Permissions: Devices may only read and write their owndeviceId corresponding bus Topic, Disable cross-device access.

2.2 Global public header

{
  "protocolVersion": "2.2",
  "messageId": "UUID-v4 Global Unique",
  "deviceId": "CAM-20260001",
  "sentAt": "2026-07-24T09:30:00Z",
  "msgDir": "platform2dev/dev2platform",
  "mainBizType": "device/aiBiz",
  "subBizType": "Segment Business Identification",
  "payload": {},
  "extend": {}
}

Public Header Field Description

| Fields | Type | Required | Description |
| --- | --- | --- | --- |
| protocolVersion | string | Yes | Protocol Version 2.2, Major version change represents incompatibility |
| messageId | string | Yes | UUID v4 idempotent ID, Retransmit / Retry must reuse the same value |
| deviceId | string | Yes | Device Global Unique Number, Topic / Message Body / Three unified platform equipment libraries |
| sentAt | string | Yes | UTC Standard Time yyyy-MM-ddTHH:mm:ssZ |
| msgDir | string | Yes | platform2dev Issued by the platform; dev2platform Device reports |
| mainBizType | string | Yes | Choose only one of the two options: device Device Hardware Class / aiBiz AI All Business Category |
| subBizType | string | Yes | Segment Business Identification, Differentiate between different functions in the same category |
| payload | object | Yes | Independent business payload, differed subBizType Field fully isolated |
| extend | object | No | Vendor proprietary hardware / Debug Extensions, Uncontaminated Criteria Fields |

2.3 Generic Unified Receipt Template

AcknowledgmentsubBizType Consistent with original request, PassedsourceMsgId Bind original message for request matching

{
  "protocolVersion": "2.2",
  "messageId": " acknowledgment    UUID",
  "deviceId": "CAM-20260001",
  "sentAt": "2026-07-24T09:30:01Z",
  "msgDir": "dev2platform",
  "mainBizType": "device",
  "subBizType": "config",
  "payload": {
    "sourceMsgId": "Original requestmessageId",
    "code": 200,
    "msg": "Business Execution Description",
    "errCode": 0,
    "errDetail": "",
    "bizData": {}
  },
  "extend": {}
}

Acknowledgment payload Common Fields

| Fields | Type | Required | Description |
| --- | --- | --- | --- |
| sourceMsgId | string | Yes | Corresponding to the order issued by messageId, Used for requests - Answer Associations |
| code | int | Yes | HTTP Alignment Status Code: 200 Success, 4xx Parameter salah, 5xx Device internal abnormality |
| msg | string | Yes | Readable Short Execution Result Copy |
| errCode | int | Yes | Custom Business Error Codes, 0 = fault-free, Non-zero subdivision fault |
| errDetail | string | No | Detailed fault stack / Reason, Successfully left blank |
| bizData | object | No | Query / Snap / List business return data, Leave blank for pure setup action |

3. Device Class

Main Business Type: mainBizType=device

3.1 Full device configuration

Sub Business Type: subBizType=config

Business Description

Issued by the platform / Query audio and video. NET. PTZ. OSD. GB28181. RTMP. Serial port. Save. MQTT Full hardware parameters, Multiplexing HTTP Interface Fields.

issued payload Fields

platform2dev

| Fields | Type | Required | Description |
| --- | --- | --- | --- |
| operate | string | Yes | get Query current configuration / set Save New Configuration |
| configGroup | string | Yes | basic/net/video/audio/ptz/osd/ai/security/storage/time/rs485/rtmp/gb28181/mqtt/serial/mp3 |
| configData | object | Yes | Grouping Corresponding Parameters, get Spatio-temporal objects{} |

Issued by the platform JSON EXAMPLE

{
  "protocolVersion": "2.2",
  "messageId": "cmd-config-00112233-4455-6677-8899-aabbccddeeff",
  "deviceId": "CAM-20260001",
  "sentAt": "2026-07-24T09:32:00Z",
  "msgDir": "platform2dev",
  "mainBizType": "device",
  "subBizType": "config",
  "payload": {
    "operate": "set",
    "configGroup": "video",
    "configData": {
      "code_stream_type": "Master Stream",
      "resolution": "1920*1080",
      "video_coding": "H.265",
      "frame_rate": 25,
      "StreamMode": "VBR",
      "MaxStreams": 2048,
      "EncodeAudio": 1,
      "AudioMode": "G711A",
      "AuidoInputVolume": 70,
      "AudioOutputVolume": 70
    }
  },
  "extend": {}
}

Configure Receipt JSON EXAMPLE

{
  "protocolVersion": "2.2",
  "messageId": "ack-config-00aabbcc-1122-3344-5566-77889900aabb",
  "deviceId": "CAM-20260001",
  "sentAt": "2026-07-24T09:32:02Z",
  "msgDir": "dev2platform",
  "mainBizType": "device",
  "subBizType": "config",
  "payload": {
    "sourceMsgId": "cmd-config-00112233-4455-6677-8899-aabbccddeeff",
    "code": 200,
    "msg": "Video code stream configuration saved successfully",
    "errCode": 0,
    "errDetail": "",
    "bizData": {}
  },
  "extend": {}
}

3.2 General device control

Sub Business Type: subBizType=ctrl

Business Description

Issuing Hardware Operating Instructions: Reboot. Reset factory. PTZ. To Format    SD. Start-stop recording. Firmware Upgrade. Parameter import and export.

issued payload Fields

| Fields | Type | Required | Description |
| --- | --- | --- | --- |
| ctrlAction | string | Yes | reboot/fully_recovery/recordStart/recordStop/ptzCmd/formatDisk/firmwareUpgrade/exportParam/importParams |
| param | object | No | Action Matching Parameters, PTZ / Format is required, Reboot is nullable |
| param.ptzCmd | string | ptz Action is required | ptz_up/ptz_down/call_preset/add_preset/delete_preset |
| param.presetId | int | Gimbal preset position is required | Preset bit number 1~36 |
| param.speed | int | Gimbal mobile is required | Movement Speed 1~36 |

Issued by the platform PTZ Example Instructions

{
  "protocolVersion": "2.2",
  "messageId": "cmd-ctrl-ptz-00998877-6655-4433-2211-abcdef987654",
  "deviceId": "CAM-20260001",
  "sentAt": "2026-07-24T09:34:00Z",
  "msgDir": "platform2dev",
  "mainBizType": "device",
  "subBizType": "ctrl",
  "payload": {
    "ctrlAction": "ptzCmd",
    "param": {
      "ptzCmd": "call_preset",
      "presetId": 1,
      "speed": 10
    }
  },
  "extend": {}
}

Examples of control receipts

{
  "protocolVersion": "2.2",
  "messageId": "ack-ctrl-ptz-11223344-5566-7788-9900-123456abcdef",
  "deviceId": "CAM-20260001",
  "sentAt": "2026-07-24T09:34:03Z",
  "msgDir": "dev2platform",
  "mainBizType": "device",
  "subBizType": "ctrl",
  "payload": {
    "sourceMsgId": "cmd-ctrl-ptz-00998877-6655-4433-2211-abcdef987654",
    "code": 200,
    "msg": "Gimbal call preset1Complete",
    "errCode": 0,
    "errDetail": "",
    "bizData": {
      "ctrlAction": "ptzCmd",
      "presetId": 1
    }
  },
  "extend": {}
}

3.3 Remote Snapshot

Sub Business Type: subBizType=snapshot

Business Description

The platform sends real-time snapshots, Device Return  Base64  Images, Hardware Media Actions.

issued payload Fields

| Fields | Type | Required | Description |
| --- | --- | --- | --- |
| ctrlAction | string | Yes | FixedSnapShot |
| type | string | Yes | FixedDirectly |
| snapChannel | int | Yes | Capture channel number |

Issuing Examples

{
  "protocolVersion": "2.2",
  "messageId": "cmd-snap-1234abcd-5678ef90-112233445566",
  "deviceId": "CAM-20260001",
  "sentAt": "2026-07-24T09:35:00Z",
  "msgDir": "platform2dev",
  "mainBizType": "device",
  "subBizType": "snapshot",
  "payload": {
    "ctrlAction": "SnapShot",
    "type": "Directly",
    "snapChannel": 1
  },
  "extend": {}
}

Capture Receipt bizData Fields

| Fields | Type | Description |
| --- | --- | --- |
| result | string | ok/failed |
| snapChannel | int | Capture channel |
| image | string | jpg base64 Coding Diagram |

Receipt full example

{
  "protocolVersion": "2.2",
  "messageId": "ack-snap-9876dcba-4321fe09-667788990011",
  "deviceId": "CAM-20260001",
  "sentAt": "2026-07-24T09:35:01Z",
  "msgDir": "dev2platform",
  "mainBizType": "device",
  "subBizType": "snapshot",
  "payload": {
    "sourceMsgId": "cmd-snap-1234abcd-5678ef90-112233445566",
    "code": 200,
    "msg": "Capture Complete",
    "errCode": 0,
    "errDetail": "",
    "bizData": {
      "result": "ok",
      "snapChannel": 1,
      "image": "data:image/jpg;base64,/9j/4AAQSkZJRgABAQAAAQABAAD/2wCEAAEBAQEBAQEBAQEB..."
    }
  },
  "extend": {}
}

3.4 Device Calibration Time

Sub Business Type: subBizType=time

Business Description

Issue a standard time synchronization device hardware clock.

issued payload Fields

| Fields | Type | Required | Description |
| --- | --- | --- | --- |
| time | string | Yes | yyyy-MM-dd HH:mm:ss |

Issuing Examples

{
  "protocolVersion": "2.2",
  "messageId": "cmd-time-22334455-66778899-00112233abcd",
  "deviceId": "CAM-20260001",
  "sentAt": "2026-07-24T09:36:00Z",
  "msgDir": "platform2dev",
  "mainBizType": "device",
  "subBizType": "time",
  "payload": {
    "time": "2026-07-24 09:36:00"
  },
  "extend": {}
}

Time Synchronization Acknowledgment Example

{
  "protocolVersion": "2.2",
  "messageId": "ack-time-33445566-77889900-11223344dcba",
  "deviceId": "CAM-20260001",
  "sentAt": "2026-07-24T09:36:00Z",
  "msgDir": "dev2platform",
  "mainBizType": "device",
  "subBizType": "time",
  "payload": {
    "sourceMsgId": "cmd-time-22334455-66778899-00112233abcd",
    "code": 200,
    "msg": "Device time synchronization completed",
    "errCode": 0,
    "errDetail": "",
    "bizData": {}
  },
  "extend": {}
}

3.5 Media Upload Interaction

Sub Business Type: subBizType=mediaUpload

Business Description

Images / Video walk MinIO HTTP Upload it, MQTT Transfer metadata only, Forbidden Large Base64.

1) The platform obtains the upload address

| Fields | Type | Required | Description |
| --- | --- | --- | --- |
| mediaId | string | Yes | Media Unique UUID |
| mediaType | string | Yes | image/jpeg / video/mp4 |
| fileSize | number | Yes | File Byte Size |
| sha256 | string | Yes | File validation hash |

Issuing Examples:

{
  "protocolVersion": "2.2",
  "messageId": "cmd-media-upload-00112233-44556677-8899aabbccdd",
  "deviceId": "CAM-20260001",
  "sentAt": "2026-07-24T09:37:00Z",
  "msgDir": "platform2dev",
  "mainBizType": "device",
  "subBizType": "mediaUpload",
  "payload": {
    "mediaId": "MEDIA-UUID-0001",
    "mediaType": "image/jpeg",
    "fileSize": 102400,
    "sha256": "abcdef1234567890abcdef1234567890"
  },
  "extend": {}
}

2) Platform Returns Upload Address Acknowledgment

| Fields | Type | Description |
| --- | --- | --- |
| mediaId | string | Media ID |
| uploadUrl | string | PUT Signature Upload Address |
| objectUrl | string | Permanent access address |
| expiresAt | string | URL Overdue UTC Time |

Receipt Example: bizData Fields

{
  "protocolVersion": "2.2",
  "messageId": "ack-media-url-11223344-55667788-9900aabbccdd",
  "deviceId": "CAM-20260001",
  "sentAt": "2026-07-24T09:37:01Z",
  "msgDir": "dev2platform",
  "mainBizType": "device",
  "subBizType": "mediaUpload",
  "payload": {
    "sourceMsgId": "cmd-media-upload-00112233-44556677-8899aabbccdd",
    "code": 200,
    "msg": "Upload address generated",
    "errCode": 0,
    "errDetail": "",
    "bizData": {
      "mediaId": "MEDIA-UUID-0001",
      "uploadUrl": "https://minio.test.com/upload/xxx",
      "objectUrl": "https://minio.test.com/res/xxx",
      "expiresAt": "2026-07-24T10:37:00Z"
    }
  },
  "extend": {}
}

3) Device Upload Completion Report

| Fields | Type | Description |
| --- | --- | --- |
| mediaId | string | Media ID |
| objectUrl | string | Permanent Resource Address |
| fileSize | number | File &Size |
| sha256 | string | File Hash |
| uploadCostMs | number | Upload time in milliseconds |

Escalate full example:

{
  "protocolVersion": "2.2",
  "messageId": "up-media-finish-22334455-66778899-0011aabbccdd",
  "deviceId": "CAM-20260001",
  "sentAt": "2026-07-24T09:37:45Z",
  "msgDir": "dev2platform",
  "mainBizType": "device",
  "subBizType": "mediaUpload",
  "payload": {
    "mediaId": "MEDIA-UUID-0001",
    "objectUrl": "https://minio.test.com/res/xxx",
    "fileSize": 102400,
    "sha256": "abcdef1234567890abcdef1234567890",
    "uploadCostMs": 800
  },
  "extend": {}
}

3.6 Device heartbeat & Hardware telemetry

Sub Business Type: subBizType=state

Business Description

The device periodically reports its online status. Hardware resources. Version, MESSAGE    Retain=true, Disconnect the network and automatically send the testament offline message.

Report payload Fields

| Fields | Type | Description |
| --- | --- | --- |
| online | bool | true Online /false Offline |
| reason | string | normal/mqtt_connection_lost/power_off |
| heartbeatIndex | int | Heartbeat Count, Restart Reset 1 |
| deviceName | string | Equipment Name |
| deviceSerial | string | Hardware serial |
| version | string | Firmware Version |
| deviceFaceVer | string | Face Library Version (Face Machine Exclusive) |
| ipAddr | string | LAN IP |
| mac | string | MAC <g id="Bold">Address:</g> |
| telemetry | object | Hardware resources |
| telemetry.cpu | int | CPU Occupied % |
| telemetry.mem | int | Memory used: % |
| telemetry.diskUsed | int | Disk usage % |
| telemetry.diskTotalGB | int | Total Disk Capacity |
| telemetry.temp | int | Device Temperature℃ |
| telemetry.netUpMbps | float | Upstream bandwidth |
| telemetry.netDownMbps | float | Downstream bandwidth |
| serviceStatus | object | Background service status |
| serviceStatus.rtsp | bool | RTSP Service |
| serviceStatus.gb28181 | bool | GB28181 Service |
| serviceStatus.aiInfer | bool | AI Reasoning Service |

Complete Normal Heartbeat Report Example

{
  "protocolVersion": "2.2",
  "messageId": "up-heartbeat-33445566-77889900-1122aabbccdd",
  "deviceId": "CAM-20260001",
  "sentAt": "2026-07-24T09:38:00Z",
  "msgDir": "dev2platform",
  "mainBizType": "device",
  "subBizType": "state",
  "payload": {
    "online": true,
    "reason": "normal",
    "heartbeatIndex": 120,
    "deviceName": "Park East Gate Face Machine",
    "deviceSerial": "3161w316156d33x966",
    "version": "v1.1.02",
    "deviceFaceVer": "1021_v2",
    "ipAddr": "192.168.1.100",
    "mac": "00:11:22:33:44:55",
    "telemetry": {
      "cpu": 32,
      "mem": 45,
      "diskUsed": 68,
      "diskTotalGB": 1024,
      "temp": 48,
      "netUpMbps": 8.2,
      "netDownMbps": 12.5
    },
    "serviceStatus": {
      "rtsp": true,
      "gb28181": false,
      "aiInfer": true
    }
  },
  "extend": {}
}

Examples of offline will messages

{
  "protocolVersion": "2.2",
  "messageId": "up-will-offline-44556677-88990011-2233aabbccdd",
  "deviceId": "CAM-20260001",
  "sentAt": "2026-07-24T09:38:50Z",
  "msgDir": "dev2platform",
  "mainBizType": "device",
  "subBizType": "state",
  "payload": {
    "online": false,
    "reason": "mqtt_connection_lost",
    "heartbeatIndex": 120,
    "deviceName": "Park East Gate Face Machine",
    "deviceSerial": "3161w316156d33x966",
    "version": "v1.1.02",
    "ipAddr": "192.168.1.100",
    "mac": "00:11:22:33:44:55",
    "telemetry": {},
    "serviceStatus": {}
  },
  "extend": {}
}

}

3.8 QR Code Recognition Report

Sub Business Type: subBizType=qrcode

Business Description

Camera Hardware Recognition QR Code Active Push Content.

Report payload Fields

| Fields | Type | Description |
| --- | --- | --- |
| operator | string | FixedQRCodePush |
| info.facesluiceId | string | Equipment ID |
| info.time | string | Recognition time yyyy-MM-dd HH:mm:ss |
| info.QRCodeInfo | string | QR code original text |

Escalate full example:

{
  "protocolVersion": "2.2",
  "messageId": "up-qrcode-scan-33445566-77889900-1122aabbccdd",
  "deviceId": "CAM-20260001",
  "sentAt": "2026-07-24T09:44:00Z",
  "msgDir": "dev2platform",
  "mainBizType": "device",
  "subBizType": "qrcode",
  "payload": {
    "operator": "QRCodePush",
    "info": {
      "facesluiceId": "CAM-20260001",
      "time": "2026-07-24 09:44:00",
      "QRCodeInfo": "https://test.company.com/user/10001"
    }
  },
  "extend": {}
}

3.10 Local Video Query

Sub Business Type: subBizType=recordSearch

Business Description

Platform release period, Device Return SD Card Recording File List.

issued payload Fields

| Fields | Type | Required | Description |
| --- | --- | --- | --- |
| startTime | string | Yes | yyyy-MM-dd HH:mm:ss |
| endTime | string | Yes | yyyy-MM-dd HH:mm:ss |

Issuing Examples:

{
  "protocolVersion": "2.2",
  "messageId": "cmd-record-search-44556677-88990011-2233aabbccdd",
  "deviceId": "CAM-20260001",
  "sentAt": "2026-07-24T09:45:00Z",
  "msgDir": "platform2dev",
  "mainBizType": "device",
  "subBizType": "recordSearch",
  "payload": {
    "startTime": "2026-07-24 08:00:00",
    "endTime": "2026-07-24 09:00:00"
  },
  "extend": {}
}

Inquiry Receipt bizData Fields

| Fields | Type | Description |
| --- | --- | --- |
| facesluiceId | string | Equipment ID |
| result | string | ok/failed |
| filelist | array | Recording File Array |
| filelist.filename | string | Local file path |
| filelist.type | int | Recording type |
| filelist.start_time | long | Started timestamp |
| filelist.end_time | long | End Timestamp |

Receipt full example:

{
  "protocolVersion": "2.2",
  "messageId": "ack-record-search-55667788-99001122-3344aabbccdd",
  "deviceId": "CAM-20260001",
  "sentAt": "2026-07-24T09:45:01Z",
  "msgDir": "dev2platform",
  "mainBizType": "device",
  "subBizType": "recordSearch",
  "payload": {
    "sourceMsgId": "cmd-record-search-44556677-88990011-2233aabbccdd",
    "code": 200,
    "msg": "Video file query complete",
    "errCode": 0,
    "errDetail": "",
    "bizData": {
      "filelist": [
        {
          "filename": "/sdcard/20260724/N165617.mp4",
          "type": 1,
          "start_time": 1784966400,
          "end_time": 1784969400
        }
      ],
      "facesluiceId": "CAM-20260001",
      "result": "ok"
    }
  },
  "extend": {}
}

3.11 Video upload task release

Sub Business Type: subBizType=recordUpload

Business Description

Post Recording HTTP Upload Task, Device Binary POST Upload file.

issued payload Fields

| Fields | Type | Required | Description |
| --- | --- | --- | --- |
| startTime | string | Yes | Recording start time |
| endTime | string | Yes | Recording End Time |
| filename | string | Yes | Local Recording Path |
| uploadurl | string | Yes | Back-end reception POST <g id="Bold">Address:</g> |
| videoId | string | No | Custom Recording Unique Identification |

Issuing Examples:

{
  "protocolVersion": "2.2",
  "messageId": "cmd-record-upload-66778899-00112233-4455aabbccdd",
  "deviceId": "CAM-20260001",
  "sentAt": "2026-07-24T09:45:30Z",
  "msgDir": "platform2dev",
  "mainBizType": "device",
  "subBizType": "recordUpload",
  "payload": {
    "startTime": "2026-07-24 08:00:00",
    "endTime": "2026-07-24 09:00:00",
    "filename": "/sdcard/20260724/N165617.mp4",
    "uploadurl": "",
    "videoId": "VID-0001"
  },
  "extend": {}
}

3.13 Hardware information query

Sub Business Type: subBizType=systemInfo

Business Description

Query SD Card / 4G/WiFi/ Firmware Version Hardware Information.

issued payload Fields

| Fields | Type | Required | Description |
| --- | --- | --- | --- |
| queryType | string | Yes | GetSdCardInfo/Get4GStatus/GetWifiStatus/GetDeviceVersion |

Issuing Examples (Query SD Card) :

{
  "protocolVersion": "2.2",
  "messageId": "cmd-system-sd-77889900-11223344-5566aabbccdd",
  "deviceId": "CAM-20260001",
  "sentAt": "2026-07-24T09:47:00Z",
  "msgDir": "platform2dev",
  "mainBizType": "device",
  "subBizType": "systemInfo",
  "payload": {
    "queryType": "GetSdCardInfo"
  },
  "extend": {}
}

SD Card Inquiry Receipt bizData Fields

| Fields | Type | Description |
| --- | --- | --- |
| sd_info.status | int | 0 No card / 1 unformatted / 3 Mount / 5 In Use / 6 Full |
| sd_info.total_size | int | Total capacity MB |
| sd_info.free_size | int | volumes remaining MB |
| sd_info.record_mode | int | 0 Stop\tF6 / 1 Whole Days / 2 Alarm Recording |
| sd_info.record_filelen | int | Video Slicing Minutes |

Receipt full example:

{
  "protocolVersion": "2.2",
  "messageId": "ack-system-sd-88990011-22334455-6677aabbccdd",
  "deviceId": "CAM-20260001",
  "sentAt": "2026-07-24T09:47:00Z",
  "msgDir": "dev2platform",
  "mainBizType": "device",
  "subBizType": "systemInfo",
  "payload": {
    "sourceMsgId": "cmd-system-sd-77889900-11223344-5566aabbccdd",
    "code": 200,
    "msg": "SDCard information read successfully",
    "errCode": 0,
    "errDetail": "",
    "bizData": {
      "sd_info": {
        "status": 3,
        "total_size": 8192,
        "free_size": 1024,
        "record_mode": 2,
        "record_filelen": 5
      }
    }
  },
  "extend": {}
}

3.14 Device Log Paginated Report

Sub Business Type: subBizType=log

Business Description

Equipment operation. Operation Audit Log Paging Push.

Report payload Full Field

| Fields | Type | Description |
| --- | --- | --- |
| page | int | page |
| limit | int | Number of bars per page |
| total | int | Total Log Entries |
| logList | array | Log Detail Array |
| logList.serial | int | Log Sequence Number |
| logList.time | string | Log Time |
| logList.mainType | string | Log Main Category: &Normal / Abnormal |
| logList.subType | string | Segmentation Fault / OPERATIONS |
| logList.channel | int | Corresponding channel |
| logList.user | string | Action User |
| logList.remoteIp | string | Actions Client IP |

Escalate full example:

{
  "protocolVersion": "2.2",
  "messageId": "up-log-page-66778899-00112233-4455aabbccdd",
  "deviceId": "CAM-20260001",
  "sentAt": "2026-07-24T09:46:00Z",
  "msgDir": "dev2platform",
  "mainBizType": "device",
  "subBizType": "log",
  "payload": {
    "page": 1,
    "limit": 10,
    "total": 120,
    "logList": [
      {
        "serial": 2,
        "time": "2026/07/24 09:45:00",
        "mainType": "Abnormal",
        "subType": "Abnormal audio input",
        "channel": 1,
        "user": "admin",
        "remoteIp": "192.168.1.100"
      }
    ]
  },
  "extend": {}
}

4. AI Business type

Main Business Type: mainBizType=aiBiz

4.1 AI Model Issue Deployment

Sub Business Type: subBizType=modelDeploy

Business Description

AI  Reasoning Model Download. Correction. Toggle inference instance.

issued payload Full Field

| Fields | Type | Required | Description |
| --- | --- | --- | --- |
| requestId | string | Yes | Model Task Unique ID |
| algorithmId | int | Yes | Algorithm Business Number |
| trainingId | string | No | Task ID |
| modelType | string | Yes | om/tensorrt |
| modelUrl | string | Yes | Short-term signatures HTTP Download url |
| fileName | string | Yes | Model Filename |
| fileSize | number | Yes | File Byte Size |
| sha256 | string | Yes | File hash checksum |
| expiresAt | string | Yes | Download link expired UTC Time |
| rollbackEnable | bool | Yes | Failed Automatic Rollback of Old Model |
| modelConfig | object | No | Inference Parameter Threshold |
| modelConfig.confThreshold | float | Confidence threshold |  |
| modelConfig.nmsThreshold | float | NMS Suppression Threshold |  |

Issue full example:

{
  "protocolVersion": "2.2",
  "messageId": "cmd-model-77886655-1234-5678-abcd-12345678abcd",
  "deviceId": "CAM-20260001",
  "sentAt": "2026-07-24T09:33:00Z",
  "msgDir": "platform2dev",
  "mainBizType": "aiBiz",
  "subBizType": "modelDeploy",
  "payload": {
    "requestId": "MODEL-TASK-20260724-001",
    "algorithmId": 1001,
    "trainingId": "TRAIN-0001",
    "modelType": "om",
    "modelUrl": "https://minio.test.com/ai/model/detect.om",
    "fileName": "detect.om",
    "fileSize": 12580000,
    "sha256": "3f2c9d11e88a77b665443211abcdef00987654321",
    "expiresAt": "2026-07-25T00:00:00Z",
    "rollbackEnable": true,
    "modelConfig": {
      "confThreshold": 0.5,
      "nmsThreshold": 0.4
    }
  },
  "extend": {}
}

Model Receipt bizData Fields

| Fields | Type | Description |
| --- | --- | --- |
| requestId | string | Corresponding issue task ID |
| status | string | RECEIVED/DOWNLOADING/DOWNLOADED/VERIFYING/DEPLOYING/SUCCESS/FAILED |
| fileSha256 | string | Local Validation Hash |
| costMs | number | Deployment takes milliseconds |

Receipt full example:

{
  "protocolVersion": "2.2",
  "messageId": "ack-model-11223344-5566-7788-9900-abcdef123456",
  "deviceId": "CAM-20260001",
  "sentAt": "2026-07-24T09:33:45Z",
  "msgDir": "dev2platform",
  "mainBizType": "aiBiz",
  "subBizType": "modelDeploy",
  "payload": {
    "sourceMsgId": "cmd-model-77886655-1234-5678-abcd-12345678abcd",
    "code": 200,
    "msg": "Model validation passed, Deployment Completed",
    "errCode": 0,
    "errDetail": "",
    "bizData": {
      "requestId": "MODEL-TASK-20260724-001",
      "status": "SUCCESS",
      "fileSha256": "3f2c9d11e88a77b665443211abcdef00987654321",
      "costMs": 1200
    }
  },
  "extend": {}
}

4.2 Query current loading model

Sub Business Type: subBizType=modelQuery

Business Description

Query device is currently active AI Models, issued payload is an empty object{} Issuing Examples:

{
  "protocolVersion": "2.2",
  "messageId": "cmd-model-query-11112222-33334444-55556666",
  "deviceId": "CAM-20260001",
  "sentAt": "2026-07-24T10:00:00Z",
  "msgDir": "platform2dev",
  "mainBizType": "aiBiz",
  "subBizType": "modelQuery",
  "payload": {},
  "extend": {}
}

Inquiry Receipt bizData Fields

algorithmId、fileName、sha256、modelType

4.3 Model Manual Rollback

Sub Business Type: subBizType=modelRollback

issued payload Fields

| Fields | Type | Required | Description |
| --- | --- | --- | --- |
| requestId | string | Yes | Rollback Task ID |

4.4 Face Library People Management

Sub Business Type: subBizType=faceLib

Business Description

Addition, deletion and modification of face base database (Add / Delete / Pagination Query / Single query / Empty) .

issued payload Common Fields

| Fields | Type | Required | Description |
| --- | --- | --- | --- |
| libOperate | string | Yes | EditPerson/QueryPerson/QuerySample/SearchPerson/DelPerson/DeleteAllPerson |
| customId | string | Addition or deletion of check list is required | Person Globally Unique UUID |
| name | string | EditPerson Required | Name of the person |
| telnum1 | string | Optional | Phone |
| age | int | Optional | Age |
| gender | int | 0 men / 1 Female |  |
| idCard | string | Optional ID |  |
| valid_time_type | int | 0 Daily / 1 week / 2 Date Range |  |
| start_time | long | Valid from seconds |  |
| expire_time | long | Expiration Seconds |  |
| department_name | string | Department |  |
| personType | int | 0 White / 1 black / 2VIP |  |
| notes | string | Remarks |  |
| op_face_ver | string | Face Library Version |  |
| pic | string | Face Bottom Chart base64 |  |

Full example of a new person posting

{
  "protocolVersion": "2.2",
  "messageId": "cmd-face-lib-add-77889900-11223344-5566aabbccdd",
  "deviceId": "CAM-20260001",
  "sentAt": "2026-07-24T09:40:00Z",
  "msgDir": "platform2dev",
  "mainBizType": "aiBiz",
  "subBizType": "faceLib",
  "payload": {
    "libOperate": "EditPerson",
    "customId": "713BCEF6393955E0DC8822354D0D61E1",
    "name": " Pcs 3 ",
    "telnum1": "13800138000",
    "age": 30,
    "gender": 0,
    "idCard": "400400199912120001",
    "valid_time_type": 0,
    "start_time": 0,
    "expire_time": 86399,
    "department_name": "Administration Department",
    "personType": 0,
    "notes": "Employees within the campus",
    "op_face_ver": "1021_v2",
    "pic": "data:image/jpg;base64,/9j/4AAQSkZJRgABAQAAAQABAAD..."
  },
  "extend": {}
}

Face Library Operation Receipt Example

{
  "protocolVersion": "2.2",
  "messageId": "ack-face-lib-add-88990011-22334455-6677aabbccdd",
  "deviceId": "CAM-20260001",
  "sentAt": "2026-07-24T09:40:02Z",
  "msgDir": "dev2platform",
  "mainBizType": "aiBiz",
  "subBizType": "faceLib",
  "payload": {
    "sourceMsgId": "cmd-face-lib-add-77889900-11223344-5566aabbccdd",
    "code": 200,
    "msg": "Person added successfully",
    "errCode": 0,
    "errDetail": "",
    "bizData": {
      "customId": "713BCEF6393955E0DC8822354D0D61E1",
      "personId": 1,
      "result": "ok"
    }
  },
  "extend": {}
}

4.5 Face passage / Stranger Snap Report

Sub Business Type: subBizType=faceEvent

Business Description

faces AI Recognition Result Report, Distinguish whitelisted passes / stranger.

Report payload Full Field

| Fields | Type | Description |
| --- | --- | --- |
| operator | string | entr Whitelist /entr_or_exit stranger |
| info.customId | string | People in the Library ID, Stranger Empty |
| info.personid | int | Device Local Personnel Number |
| info.persionName | string | Name |
| info.facesluiceId | string | Equipment ID |
| info.time | string | Snapping time yyyy-MM-dd HH:mm:ss |
| info.pic | string | Live Snapshot base64 |
| info.sample_pic | string | Diku face map base64 |
| info.score_dect | int | Comparison Score 0~100 |
| info.rect | array | [x,y,w,h] Face Frame Coordinates |
| info.gender | string | M men / F Female |
| info.glasses | int | 1 None / 2 Normal / 3 sunglasses |
| info.mask | int | 1 None / 2 Wear a mask |
| info.hat | int | 1 None / 2 Wear a hat |

Escalate full example:

{
  "protocolVersion": "2.2",
  "messageId": "up-face-pass-99001122-33445566-7788aabbccdd",
  "deviceId": "CAM-20260001",
  "sentAt": "2026-07-24T09:41:00Z",
  "msgDir": "dev2platform",
  "mainBizType": "aiBiz",
  "subBizType": "faceEvent",
  "payload": {
    "operator": "entr",
    "info": {
      "customId": "713BCEF6393955E0DC8822354D0D61E1",
      "personid": 1,
      "persionName": "Zhang San",
      "facesluiceId": "CAM-20260001",
      "time": "2026-07-24 09:41:00",
      "pic": "data:image/jpg;base64,xxx",
      "sample_pic": "data:image/jpg;base64,xxx",
      "score_dect": 92,
      "rect": [100, 120, 220, 260],
      "gender": "M",
      "glasses": 1,
      "mask": 1,
      "hat": 1
    }
  },
  "extend": {}
}

4.6 Structured human carriage / Plate Number / Non-motor vehicle identification

Sub Business Type: subBizType=struct

Business Description

Common Objectives AI Track and escalate, faces / Figurine / motor vehicle / Non-Vehicle / License plate unified structure.

Report payload Full Field

| Fields | Type | Description |
| --- | --- | --- |
| operator | string | Fixedstruct_attr |
| facesluiceId | string | Equipment ID |
| track_id | int | Target Tracking Unique ID |
| type | string | face/human/vehicle/cycle/plate |
| keep_time | int | Video stays in milliseconds |
| left/top/right/bottom | int | Target Pixel Box |
| worth | float | Identify confidence 0~1 |
| datetime | string | Snapping time |
| bind | array | binding target track_id (Face bound humanoid) |
| info | object | Target Segment Properties |
| object_image | string | Target screenshot base64 |
| bg_image | string | Background Image base64 |
| mac | string | Equipment MAC |
| ipaddr | string | Equipment IP |

Complete Structured Human Report Example

{
  "protocolVersion": "2.2",
  "messageId": "up-struct-human-00112233-44556677-8899aabbccdd",
  "deviceId": "CAM-20260001",
  "sentAt": "2026-07-24T09:42:00Z",
  "msgDir": "dev2platform",
  "mainBizType": "aiBiz",
  "subBizType": "struct",
  "payload": {
    "operator": "struct_attr",
    "facesluiceId": "CAM-20260001",
    "track_id": 4,
    "type": "human",
    "keep_time": 2022,
    "left": 783,
    "top": 443,
    "right": 1272,
    "bottom": 983,
    "worth": 0.853293,
    "datetime": "2026-07-24 09:42:00",
    "bind": [],
    "info": {
      "clothes_color": 4,
      "safety_helmet": 1,
      "action_watch_phone": 1,
      "bags": 1
    },
    "object_image": "data:image/jpeg;base64,xxx",
    "bg_image": "data:image/jpeg;base64,xxx",
    "mac": "00:11:22:33:44:55",
    "ipaddr": "192.168.1.100"
  },
  "extend": {}
}

5. IoT Center Type

Main Business Type: mainBizType=RS

5.1 RSGet

Sub Business Type: subBizType=RSGet

Business Description

GET AcquireRS-485Configure

Request Parameter

| Name | Position | Type | Required | Description |
| --- | --- | --- | --- | --- |
| accessToken | header | string | Yes | none |

Back to examples

200 Response

{

"code": 200, "message": "Success", "data": {

"baud_rate": 115200,

"data_bit": 16,

"stop_bit": 1, "check": "None",

"flow_contrl": "None",

"decoder_type": "PWMC-AE", "decoder_addr": "127.0.0.1"

}

}

Back to results

| State Code | Status code meaning | Description | Data Model |
| --- | --- | --- | --- |
| 200 | OK | none | Inline |

Back to Data Structure

State Code 200

| Name | Type | Required | bound | Display name | Description |
| --- | --- | --- | --- | --- | --- |
| » code | integer | true | none |  | none |
| » message | string | true | none |  | none |
| » data | object | true | none |  | none |
| »» baud_rate | integer | true | none | Baud rate | none |
| »» data_bit | integer | true | none | Data bit | none |
| »» stop_bit | integer | true | none | Stop bit | none |
| »» check | string | true | none | Parity | none |
| »» flow_contrl | string | true | none | Flow Control | none |
| »» decoder_type | string | true | none | Decoder Type | none |
| »» decoder_addr | string | true | none | Decoder Address | none |

5.2 RSSave

Sub Business Type: subBizType=RSSave

Business Description

POST SAVINGRS-485Configure

Request Parameter

{

"baud_rate": 115200,

"data_bit": 16,

"stop_bit": 1, "check": "None",

"flow_contrl": "None",

"decoder_type": "PWMC-AE", "decoder_addr": "127.0.0.1"

}

Request Parameter

| Name | Position | Type | Required | Description |
| --- | --- | --- | --- | --- |
| accessToken | header | string | Yes | none |
| body | body | object | No | none |

Back to examples

200 Response

{

"code": 200,

"message": "Success"

}

Back to results

| State Code | Status code meaning | Description | Data Model |
| --- | --- | --- | --- |
| 200 | OK | none | Inline |

Back to Data Structure

State Code 200

| Name | Type | Required | bound | Display name | Description |
| --- | --- | --- | --- | --- | --- |
| » code | integer | true | none |  | none |
| » message | string | true | none |  | none |

5.3 Elevator control

Sub Business Type: subBizType=tkControl

Business Description

Gate hardware Elevator floor control command issued.

issued payload Fields

| Fields | Type | Required | Description |
| --- | --- | --- | --- |
| operator | string | Yes | FixedTKControl |
| info.value | string | Yes | 32 bit hexadecimal ladder control script |

Issuing Examples:

{
  "protocolVersion": "2.2",
  "messageId": "cmd-tk-ctrl-11223344-55667788-9900aabbccdd",
  "deviceId": "CAM-20260001",
  "sentAt": "2026-07-24T09:43:00Z",
  "msgDir": "platform2dev",
  "mainBizType": "device",
  "subBizType": "tkControl",
  "payload": {
    "operator": "TKControl",
    "info": {
      "value": "FF010000000000000000"
    }
  },
  "extend": {}
}

Elevator Control Acknowledgment Example

{
  "protocolVersion": "2.2",
  "messageId": "ack-tk-ctrl-22334455-66778899-0011aabbccdd",
  "deviceId": "CAM-20260001",
  "sentAt": "2026-07-24T09:43:01Z",
  "msgDir": "dev2platform",
  "mainBizType": "device",
  "subBizType": "tkControl",
  "payload": {
    "sourceMsgId": "cmd-tk-ctrl-11223344-55667788-9900aabbccdd",
    "code": 200,
    "msg": "Elevator control command issued successfully",
    "errCode": 0,
    "errDetail": "",
    "bizData": {
      "result": "ok"
    }
  },
  "extend": {}

5.4 LCD Advertising Management

Sub Business Type: subBizType=ad

Business Description

Addition and deletion of advertisement images on face machine with screen. Carousel Duration Configuration.

issued payload Fields

| Fields | Type | Required | Description |
| --- | --- | --- | --- |
| libOperate | string | Yes | EditAD Add Amendment / DelAD Delete |
| info.adslot | int | Yes | Ad Slot 0~4 |
| info.path | string | EditAD Required | Advertising image download URL |
| info.polltime | int | EditAD Required | Single Carousel Duration (seconds) |

Issuing Examples (Add new ad) :

{
  "protocolVersion": "2.2",
  "messageId": "cmd-ad-add-44556677-88990011-2233aabbccdd",
  "deviceId": "CAM-20260001",
  "sentAt": "2026-07-24T09:44:30Z",
  "msgDir": "platform2dev",
  "mainBizType": "device",
  "subBizType": "ad",
  "payload": {
    "libOperate": "EditAD",
    "info": {
      "adslot": 0,
      "path": "https://minio.test.com/ad/ad01.jpg",
      "polltime": 10
    }
  },
  "extend": {}
}

Ad action receipt example

{
  "protocolVersion": "2.2",
  "messageId": "ack-ad-add-55667788-99001122-3344aabbccdd",
  "deviceId": "CAM-20260001",
  "sentAt": "2026-07-24T09:44:31Z",
  "msgDir": "dev2platform",
  "mainBizType": "device",
  "subBizType": "ad",
  "payload": {
    "sourceMsgId": "cmd-ad-add-44556677-88990011-2233aabbccdd",
    "code": 200,
    "msg": "Ad configuration updated successfully",
    "errCode": 0,
    "errDetail": "",
    "bizData": {
      "adslot": 0,
      "result": "ok"
    }
  },
  "extend": {}
}

5.5 MP3 Audio Playback Configuration

Sub Business Type: subBizType=mp3Play

Business Description

Device Local Audio File. Play Mode Configuration.

issued payload Fields

| Fields | Type | Required | Description |
| --- | --- | --- | --- |
| operate | string | Yes | Set settings.ini / Get Query |
| config.enable | int | Yes | 0 Off / 1 Activate the LCC |
| config.mode | int | Yes | 0 Sequence / 1 Sort Random |
| config.mp3file | array | No | Audio Galleries |
| mp3file.name | string |  | Audio Name |
| mp3file.select | int |  | 0 None / 1 Select Playback |

Issuing Examples:

{
  "protocolVersion": "2.2",
  "messageId": "cmd-mp3-set-77889900-11223344-5566aabbccdd",
  "deviceId": "CAM-20260001",
  "sentAt": "2026-07-24T09:46:00Z",
  "msgDir": "platform2dev",
  "mainBizType": "device",
  "subBizType": "mp3Play",
  "payload": {
    "operate": "Set",
    "config": {
      "enable": 1,
      "mode": 0,
      "mp3file": [
        {
          "name": "Welcome Voice.mp3",
          "select": 1
        },
        {
          "name": "Alert Note.mp3",
          "select": 0
        }
      ]
    }
  },
  "extend": {}
}

6. Global Reliability & General Safety Specifications

    - Idempotent Deduplication: per messagemessageId Global Unique, Platform todeviceId+messageId Built-in SVG, Duplicate messages discarded directly;

    - Ladder Retry: No correspondence received ack Auto Mode 5/15/30/60s Retreat, Longest retry 5 Menit; Face Library Batch Task Extended to 10

Menit;

    - Offline Cache: Device local persistence  AI   Snap. Face Library Tasks. Media Tasks, Network recovery replenished in chronological order;

    - Binary Constraints: Single Bar JSON Max 128KB, Models / Video Recording / Large image forbidden Base64 Inline, Unified HTTP/MinIO Upload it;

    - Three-factor verification: MQTT Username. Topic and within deviceId. Message Body deviceId All three must be exactly the same, Discard inconsistently and log security;

    - Transmission encryption: Production Environment Mandatory TLS 8883, 1883 Plaintext Intranet Test Only;

    - Sensitive Desensitization: <g id="Bold">ID Card</g>. Key. Temporary Upload    URL. Face original image supports configuration desensitization output;